# The Visible Computer — INT-1540

Week 2 · Module 2: The Visible Computer

Name (last name and class year only): ___________________________________

This companion enhances the PGCC curriculum; it does not replace it. Canvas is the official record of your grade.

Students with a 504 plan or IEP: your accommodations apply through PGCC Disability Support Services.

## Objectives

• Identify ATX, microATX, and mini-ITX form factors and their use cases

• Name and describe rear I/O panel ports by color and connector shape

• Distinguish USB standards by speed and port color

• Identify display connector types and their signal types

## Key Terms

ATX

305x244 mm — standard full-size desktop motherboard form factor.

I/O shield

Stamped metal plate at the rear of the case covering the motherboard's external ports.

USB 3.0

5 Gbps USB standard — identified by blue port color.

HDMI

Digital AV connector carrying both video and multi-channel audio.

VGA

Legacy 15-pin analog video connector — no audio, no digital signal.

## The Concept

Every desktop motherboard follows a form factor standard that defines physical size and mounting holes. ATX (305x244 mm) is the most common — it fits in standard desktop towers and offers the most expansion. microATX (244x244 mm) trades slots for size. mini-ITX (170x170 mm) has one PCIe slot and fits very compact cases. The rear I/O panel has a fixed set of ports: USB (color-coded by generation), display outputs (VGA/DVI/HDMI/DisplayPort), audio jacks (colored by function), and RJ-45 Ethernet. USB port color is the A+ exam standard — know it cold.

## Worked Examples

Example 1:

[object Object]

Example 2:

[object Object]

## Common Mistakes to Avoid

• Using the motherboard HDMI port when a discrete GPU is installed — this runs integrated graphics, not the GPU.

• Assuming USB-C is always fast — a USB-C port may be limited to USB 2.0 on budget hardware.

• Mixing up audio port colors: green = line-out, pink = mic, blue = line-in.

## My Notes

Use this space to write notes during class. Focus on what your facilitator emphasizes.

## Self-Check Questions

Cover the answers below. Try each question before reading the answer.

1. A blue USB port on the rear panel transfers data at what maximum speed?

• A. 5 Gbps (USB 3.0 / 3.2 Gen 1)

• B. 480 Mbps (USB 2.0)

• C. 10 Gbps (USB 3.1 Gen 2, teal)

• D. 40 Gbps (USB4, USB-C)

2. Which single cable carries both audio and video in HD quality?

• A. HDMI

• B. DVI-D

• C. VGA

• D. DisplayPort without adapter

3. mini-ITX cases typically hold how many full-size expansion cards?

• A. One

• B. Two

• C. Four

• D. Six

4. What does the I/O shield do?

• A. Covers rear panel ports and labels their locations

• B. Provides primary motherboard EMI shielding

• C. Mounts the motherboard to the rear panel

• D. Grounds the motherboard through the case

5. DVI-D transmits which signal type?

• A. Digital video only

• B. Analog video only

• C. Both analog and digital video

• D. Digital video and audio

## Self-Check Answers

1. A — Blue identifies USB 3.0 (USB 3.2 Gen 1) at 5 Gbps.

2. A — HDMI carries digital video and multi-channel audio over one cable.

3. A — mini-ITX boards have one PCIe slot — matching the single expansion slot in mini-ITX cases.

4. A — The I/O shield covers the irregular rear opening and labels each port group.

5. A — DVI-D (Digital) carries only a pure digital signal — the D suffix means digital-only.