# The CPU — INT-1540

Week 3 · Module 3: The CPU

Name (last name and class year only): ___________________________________

This companion enhances the PGCC curriculum; it does not replace it. Canvas is the official record of your grade.

Students with a 504 plan or IEP: your accommodations apply through PGCC Disability Support Services.

## Objectives

• Identify Intel (LGA) and AMD (PGA/AM4) socket types

• Explain TDP, cache hierarchy, and clock speed

• Perform correct CPU installation and thermal paste application

• Interpret common CPU troubleshooting symptoms

## Key Terms

LGA

Land Grid Array — motherboard has the pins; CPU has flat pads (Intel desktop).

PGA

Pin Grid Array — CPU has the pins; socket has holes (AMD AM4).

TDP

Thermal Design Power — max sustained watts of heat the cooler must dissipate.

L1 Cache

Fastest per-core cache (~256 KB); lowest latency, built into each core.

Thermal paste

Compound filling microscopic gaps between CPU IHS and cooler base.

## The Concept

The CPU fetches instructions from RAM, decodes them, executes operations, and writes results back. Modern CPUs have multiple cores — each capable of independent execution — plus SMT (Hyper-Threading on Intel), which presents two logical threads per physical core. Cache levels store frequently used data close to execution: L1 is fastest (per-core, ~256 KB), L2 is mid-tier (per-core, ~1-4 MB), L3 is shared (16-64 MB). TDP tells you the minimum cooler capacity in watts. Thermal paste eliminates air gaps between the CPU's heat spreader and the cooler base — air is a poor thermal conductor.

## Worked Examples

Example 1:

[object Object]

## Common Mistakes to Avoid

• Too much thermal paste — excess spreads into the socket and can short contacts.

• Pressing an LGA CPU with force — the pins are on the board; pressure bends them.

• Forgetting the fan header — the CPU throttles immediately from overheating at boot.

## My Notes

Use this space to write notes during class. Focus on what your facilitator emphasizes.

## Self-Check Questions

Cover the answers below. Try each question before reading the answer.

1. Intel desktop CPUs use which socket type?

• A. LGA — pins on the motherboard

• B. PGA — pins on the CPU

• C. BGA — soldered to board

• D. ZIF PGA — zero-force lever

2. TDP tells you what about a CPU?

• A. Max sustained heat the cooler must dissipate

• B. Peak watt draw during turbo boost

• C. Recommended PSU wattage for the system

• D. Total lifetime energy in watt-hours

3. Thermal paste's purpose is to:

• A. Fill microscopic surface gaps to improve heat transfer

• B. Bond the cooler permanently to the IHS

• C. Lubricate cooler mounting screws

• D. Provide electrical conductivity to the cooler

4. Which cache level has the lowest latency?

• A. L1 (closest to execution units)

• B. L2 (per-core mid-tier)

• C. L3 (shared last-level)

• D. L4 / eDRAM (off-die)

5. When installing an LGA CPU, which action should NOT be performed?

• A. Pressing the CPU firmly into the socket

• B. Aligning the corner triangle marker

• C. Lowering the CPU straight down

• D. Locking the retention arm after seating

## Self-Check Answers

1. A — Intel uses LGA — contact pins are embedded in the motherboard socket.

2. A — TDP specifies the minimum cooler heat-dissipation capacity in watts under sustained load.

3. A — Thermal paste fills microscopic surface irregularities between the IHS and cooler base — displacing air which is a thermal insulator.

4. A — L1 is integrated directly into each CPU core and operates at core clock speed — ~1-5 ns latency.

5. A — Pressing an LGA CPU bends the motherboard's fragile socket pins — the retention arm provides all clamping force.