# The Power Supply — INT-1540

Week 7 · Module 7: The Power Supply

Name (last name and class year only): ___________________________________

This companion enhances the PGCC curriculum; it does not replace it. Canvas is the official record of your grade.

Students with a 504 plan or IEP: your accommodations apply through PGCC Disability Support Services.

## Objectives

• Calculate required PSU wattage with 25% headroom

• Identify PSU connectors and their target components

• Interpret 80 Plus efficiency tier ratings

• Compare modular, semi-modular, and non-modular PSUs

## Key Terms

80 Plus

Efficiency certification: baseline 80%, Bronze 82%, Gold 87%, Platinum 90%, Titanium 92%.

Modular PSU

All cables detachable — connect only needed cables for clean cable management.

EPS connector

8-pin (4+4) CPU power connector providing +12V to the VRM.

PCIe power

6+2 pin connector delivering up to 150W per connector to a GPU.

Wattage headroom

Operating the PSU at ~75% rated capacity — best efficiency and lifespan.

## The Concept

The PSU converts 120V AC to stable DC: +12V (CPU, GPU, drives), +5V (USB, legacy logic), +3.3V (RAM). The 80 Plus program certifies efficiency at three load points. Higher efficiency = less wasted heat and lower electricity bills. To size: sum all component TDPs, multiply by 1.25 for 25% headroom. Modular PSUs remove unused cables for cleaner builds. Always turn the PSU rear switch off AND unplug before working inside — standby +5VSB is live any time the cord is connected, even when the PC is off.

## Worked Examples

Example 1:

[object Object]

## Common Mistakes to Avoid

• Matching PSU wattage exactly to component TDP — leaves no headroom and stresses the PSU.

• Leaving the rear PSU switch on while working inside — +5VSB standby remains live.

• Using a non-modular PSU in a compact case — excess cables block airflow.

## My Notes

Use this space to write notes during class. Focus on what your facilitator emphasizes.

## Self-Check Questions

Cover the answers below. Try each question before reading the answer.

1. 80 Plus Gold efficiency at 50% load is at least:

• A. 87%

• B. 80%

• C. 90%

• D. 92%

2. Which connector provides +12V power to the CPU?

• A. 8-pin EPS (CPU) connector

• B. 24-pin ATX main connector

• C. 6+2 pin PCIe connector

• D. 15-pin SATA power connector

3. Total component TDP is 320W. Minimum recommended PSU wattage:

• A. 400W (320 x 1.25)

• B. 320W (exact match)

• C. 500W (320 x 1.5)

• D. 640W (320 x 2.0)

4. Modular PSU improves case airflow by:

• A. Removing unused cables that block airflow

• B. Increasing PSU fan size

• C. Routing all cables through the 24-pin

• D. Replacing flat cables with round cables

5. Before working inside a PC, the PSU rear switch should be:

• A. Off (O) — and AC cord unplugged

• B. On (I) — for chassis grounding

• C. Off (O) — cord still plugged in

• D. Either position if power button is pressed

## Self-Check Answers

1. A — 80 Plus Gold requires at least 87% efficiency at 50% load.

2. A — The 8-pin EPS (4+4-pin) CPU connector delivers +12V to the VRM specifically for CPU power.

3. A — 320W x 1.25 = 400W — 25% headroom is the standard recommendation.

4. A — Removing unneeded cables eliminates clutter that would otherwise block airflow paths.

5. A — PSU switch Off AND cord unplugged removes all voltage including +5VSB standby.