# Mass Storage Technologies — INT-1540

Week 8 · Module 8: Mass Storage Technologies

Name (last name and class year only): ___________________________________

This companion enhances the PGCC curriculum; it does not replace it. Canvas is the official record of your grade.

Students with a 504 plan or IEP: your accommodations apply through PGCC Disability Support Services.

## Objectives

• Compare HDD, SATA SSD, and NVMe SSD speed and use cases

• Explain RAID 0, 1, 5, and 10 configurations

• Identify storage interface connectors and speeds

• Calculate usable capacity for common RAID levels

## Key Terms

NVMe

Non-Volatile Memory Express — PCIe-based SSD protocol up to ~7000 MB/s.

SATA III

6 Gbps interface for HDDs and SATA SSDs (~550 MB/s real-world).

RAID 0

Striped — maximum speed, zero redundancy.

RAID 1

Mirrored — full copy on second drive, 50% usable capacity.

IOPS

Input/Output Operations Per Second — random I/O performance metric.

## The Concept

Storage spans a wide performance range. A 7200 RPM HDD reads ~150 MB/s sequential. A SATA SSD reaches ~550 MB/s — 3.6x faster. A PCIe 4.0 NVMe SSD hits ~7,000 MB/s — 46x faster than the HDD. RAID adds redundancy (RAID 1, 5, 10) or performance (RAID 0). RAID 5 is a common small-business sweet spot: 3+ drives, distributed parity, survives one failure, and better capacity efficiency than RAID 10. IOPS counts random operations per second — the key metric for OS boot drives and databases, not sequential video storage.

## Worked Examples

Example 1:

[object Object]

## Common Mistakes to Avoid

• Confusing RAID 1 usable capacity — it is always 50% of total raw.

• Treating RAID as a backup — RAID protects against drive failure only, not ransomware or accidental deletion.

• Installing an NVMe SSD in an M.2 SATA-only slot — the drive is not detected.

## My Notes

Use this space to write notes during class. Focus on what your facilitator emphasizes.

## Self-Check Questions

Cover the answers below. Try each question before reading the answer.

1. Maximum real-world sequential read speed of a SATA III SSD:

• A. ~550 MB/s

• B. ~150 MB/s

• C. ~3,500 MB/s

• D. ~7,000 MB/s

2. RAID 0 provides which combination?

• A. Maximum performance, zero redundancy

• B. Full redundancy, lower performance

• C. Parity protection, moderate speed

• D. Both redundancy and performance equally

3. RAID 1 with two 4 TB drives provides how much usable storage?

• A. 4 TB (one drive mirrored)

• B. 8 TB (all drives counted)

• C. 2 TB (25% of total)

• D. 6 TB (75% of total)

4. RAID 5 requires a minimum of how many drives?

• A. 3 drives

• B. 2 drives

• C. 4 drives

• D. 5 drives

5. NVMe SSDs use which protocol and interface?

• A. NVMe over PCIe lanes

• B. AHCI over SATA III

• C. NVMe tunneled over USB

• D. SCSI over SAS interface

## Self-Check Answers

1. A — SATA III tops out at ~550 MB/s real-world due to 6 Gbps interface overhead.

2. A — RAID 0 stripes data across all drives for maximum throughput — one drive failure destroys everything.

3. A — RAID 1 mirrors — only one drive's capacity is usable = 4 TB.

4. A — RAID 5 needs at least 3 drives — N-1 for data, one drive equivalent for distributed parity.

5. A — NVMe was designed for PCIe-attached flash — it bypasses AHCI entirely with a lower-latency command set.